Experiment 1 — SRDnCNN Baseline

Semiconductor Image Restoration

# 1. Experiment Overview

Experiment 1 establishes the first reproducible baseline for the Semiconductor Image Restoration project. The objective is to evaluate an SRDnCNN-based restoration model on degraded semiconductor inspection images and establish a quantitative reference for subsequent controlled experiments.

The experiment evaluates pixel-level reconstruction quality using PSNR, structural similarity using SSIM, perceptual similarity using LPIPS, comparison against bicubic interpolation, per-image performance variation, and representative best/worst cases.

Experiment 1 is the official frozen baseline. Subsequent experiments must be compared against this result using the same evaluation protocol.

# 2. Problem Definition

The project addresses restoration of degraded semiconductor inspection images. The paired data consists of a 128 × 128, single-channel NoisyLR image and a 256 × 256, single-channel Ground Truth image.

NoisyLR (128 × 128 × 1) → Restoration → Ground Truth (256 × 256 × 1)

The restoration task therefore combines noise/degradation removal, spatial resolution recovery, and preservation of fine image structures.

# 3. Dataset

| Dataset component |  | Count |  |
| --- | --- | --- | --- |
| Ground Truth images |  | 3200 |  |
| Training NoisyLR images |  | 3209 |  |
| Test NoisyLR images |  | 400 |  |
| Split |  | Images |  |
| Training |  | 2880 |  |
| Validation |  | 320 |  |
| Total paired training data |  | 3200 |  |
| Image | Resolution |  | Channels |
| NoisyLR | 128 × 128 |  | 1 |
| Ground Truth | 256 × 256 |  | 1 |

Training batches   : 360
Validation batches : 40
Batch size         : 8

# 4. Model

Experiment 1 uses the SRDnCNN architecture.

| Parameter | Value |
| --- | --- |
| Model | SRDnCNN |
| Input channels | 1 |
| Output channels | 1 |
| Number of features | 64 |
| Residual blocks | 8 |
| Upscale factor | 2× |
| Input resolution | 128 × 128 |
| Output resolution | 256 × 256 |

Input  : [8, 1, 128, 128]
Target : [8, 1, 256, 256]
Output : [8, 1, 256, 256]

# 5. Loss Function

Experiment 1 uses GradientL1Loss with:

lambda_gradient = 0.1

The loss combines reconstruction error with a gradient-based component. The effect of this gradient component is investigated later through a controlled loss ablation.

# 6. Training Configuration

| Parameter | Value |
| --- | --- |
| Epochs | 50 |
| Batch size | 8 |
| Optimizer | Adam |
| Learning rate | 0.0001 |
| Weight decay | 0.0 |
| Scheduler | CosineAnnealingLR |
| Random seed | 42 |
| Device | CUDA |
| GPU | NVIDIA Tesla T4 |

The experiment completed all 50 epochs successfully.

# 7. Training History

| Epoch | Train Loss | Validation Loss |
| --- | --- | --- |
| 1 | 0.055088 | 0.042023 |
| 5 | 0.040634 | 0.040324 |
| 10 | 0.039825 | 0.038745 |
| 15 | 0.039492 | 0.038300 |
| 20 | 0.038998 | 0.038560 |
| 25 | 0.038684 | 0.037689 |
| 30 | 0.038406 | 0.037741 |
| 35 | 0.038182 | 0.037349 |
| 40 | 0.038048 | 0.037270 |
| 45 | 0.037986 | 0.037190 |
| 47 | 0.037970 | 0.037183 |
| 48 | 0.037962 | 0.037181 |
| 49 | 0.037959 | 0.037178 |
| 50 | 0.037959 | 0.037178 |

Best validation loss: 0.03717783619649708 at epoch 49.

# 8. Checkpoint

Original training checkpoint:

checkpoints/best_model.pth

Verified checkpoint contents:

epoch
model_state_dict
optimizer_state_dict
loss

| Property | Value |
| --- | --- |
| Best epoch | 49 |
| Best validation loss | 0.03717783619649708 |
| Checkpoint size | 8.51 MB |

Frozen copy:

experiment_results/experiment1/experiment1_best_model.pth

# 9. Evaluation Protocol

Evaluation was performed on the complete 320-image validation set. The same validation set is used for SRDnCNN, bicubic, PSNR, SSIM, and LPIPS to ensure direct comparison.

Validation images : 320
Validation batches : 40

# 10. PSNR Results

SRDnCNN:

| Statistic | PSNR |
| --- | --- |
| Mean | 28.0357 dB |
| Median | 27.8864 dB |
| Standard deviation | 4.6697 dB |
| Minimum | 11.1695 dB |
| Maximum | 44.0977 dB |

Bicubic:

| Statistic | PSNR |
| --- | --- |
| Mean | 23.0831 dB |
| Median | 22.4052 dB |
| Standard deviation | 3.5990 dB |
| Minimum | 10.1987 dB |
| Maximum | 40.1107 dB |

Mean improvement over bicubic: +4.9526 dB.

# 11. SSIM Results

SRDnCNN:

| Statistic | SSIM |
| --- | --- |
| Mean | 0.749338 |
| Median | 0.793456 |
| Standard deviation | 0.176928 |
| Minimum | 0.197963 |
| Maximum | 0.983356 |

Bicubic:

| Statistic | SSIM |
| --- | --- |
| Mean | 0.546892 |
| Median | 0.580822 |
| Standard deviation | 0.197703 |
| Minimum | 0.124164 |
| Maximum | 0.955778 |

Mean improvement over bicubic: +0.202446.

# 12. LPIPS Results

LPIPS was evaluated using the pretrained AlexNet-based LPIPS model. The grayscale images were converted consistently to three channels before evaluation. Lower LPIPS indicates greater perceptual similarity.

SRDnCNN:

| Statistic | LPIPS |
| --- | --- |
| Mean | 0.324483 |
| Median | 0.271257 |
| Standard deviation | 0.208225 |
| Minimum | 0.043771 |
| Maximum | 0.976981 |

Bicubic:

| Statistic | LPIPS |
| --- | --- |
| Mean | 0.435090 |
| Median | 0.412678 |
| Standard deviation | 0.157307 |
| Minimum | 0.103440 |
| Maximum | 0.920526 |

Mean LPIPS improvement: 0.110606 (lower is better).

# 13. Overall Comparison

| Metric | Bicubic | SRDnCNN | Improvement |
| --- | --- | --- | --- |
| Mean PSNR | 23.0831 dB | 28.0357 dB | +4.9526 dB |
| Mean SSIM | 0.546892 | 0.749338 | +0.202446 |
| Mean LPIPS | 0.435090 | 0.324483 | −0.110606 |

Metric direction: PSNR ↑, SSIM ↑, LPIPS ↓.

# 14. Per-Image Evaluation

Per-image PSNR and SSIM values for all 320 validation images:

evaluation/experiment1_validation_metrics.csv

Per-image LPIPS values:

evaluation/experiment1_lpips.csv

Frozen copies are stored under:

experiment_results/experiment1/

# 15. Representative Cases

| Case | Index | PSNR | SSIM |
| --- | --- | --- | --- |
| Best PSNR | 219 | 44.0977 dB | 0.970090 |
| Worst PSNR | 124 | 11.1695 dB | 0.275663 |
| Best SSIM | 107 | 34.1106 dB | 0.983356 |
| Worst SSIM | 272 | 20.8467 dB | 0.197963 |

## 15.1 Best PSNR — Index 219

PSNR : 44.0977 dB
SSIM : 0.970090

The SRDnCNN reconstruction is visually close to the ground truth, with strong agreement in the major visible structures.

## 15.2 Worst PSNR — Index 124

PSNR : 11.1695 dB
SSIM : 0.275663

The reconstruction differs substantially from the ground truth. The model suppresses degradation but produces a reconstruction with noticeably different texture and intensity patterns.

## 15.3 Best SSIM — Index 107

PSNR : 34.1106 dB
SSIM : 0.983356

The major structures are preserved effectively, resulting in very high structural similarity.

## 15.4 Worst SSIM — Index 272

PSNR : 20.8467 dB
SSIM : 0.197963

The reconstructed image differs considerably from the ground truth in structural appearance and appears substantially smoother.

# 16. Initial Observations

SRDnCNN substantially improves over bicubic interpolation on the current validation set across PSNR, SSIM, and LPIPS.

The PSNR range (11.1695–44.0977 dB) and SSIM range (0.197963–0.983356) indicate substantial variation across validation samples.

Difficult cases show evidence of over-smoothing, incorrect texture reconstruction, or loss of fine structural information.

These are observations rather than confirmed causal explanations. Further controlled experiments are required before making causal claims.

# 17. Research Hypothesis for Further Experiments

Experiment 1 raises the question of whether the gradient component of GradientL1Loss contributes to structural preservation.

Does the gradient component of GradientL1Loss improve semiconductor image restoration compared with standard L1 loss?

# 18. Experiment 2 — Planned Ablation

Experiment 2 changes only the loss function.

| Setting | Experiment 1 | Experiment 2 |
| --- | --- | --- |
| Model | SRDnCNN | SRDnCNN |
| Loss | GradientL1Loss (λ=0.1) | L1Loss |
| Dataset | Same | Same |
| Train/Validation split | 2880 / 320 | 2880 / 320 |
| Batch size | 8 | 8 |
| Optimizer | Adam | Adam |
| Learning rate | 0.0001 | 0.0001 |
| Scheduler | CosineAnnealingLR | CosineAnnealingLR |
| Epochs | 50 | 50 |
| Seed | 42 | 42 |
| Evaluation | PSNR + SSIM + LPIPS | PSNR + SSIM + LPIPS |

# 19. Reproducibility

| Parameter | Value |
| --- | --- |
| Model | SRDnCNN |
| Loss | GradientL1Loss; λ=0.1 |
| Training images | 2880 |
| Validation images | 320 |
| Epochs | 50 |
| Batch size | 8 |
| Optimizer | Adam |
| Learning rate | 0.0001 |
| Weight decay | 0.0 |
| Scheduler | CosineAnnealingLR |
| Seed | 42 |
| Hardware | NVIDIA Tesla T4 |

# 20. Experiment Status

| Item | Status |
| --- | --- |
| Training | COMPLETE |
| Checkpoint | SAVED |
| PSNR | COMPLETE |
| SSIM | COMPLETE |
| LPIPS | COMPLETE |
| Bicubic comparison | COMPLETE |
| Per-image metrics | SAVED |
| Failure analysis | COMPLETE |
| Results documentation | COMPLETE |
| Overall status | FROZEN BASELINE |